SynROV Operating Guide

Firmware, Processing, Web Console, and Python AiBot

# 1. Scope

This guide describes the SynROV repository according to the current code under firmware/ and software/. The public protocol, configuration, and AiBot reference is V1, represented by version: 1 and the V1 schemas listed below.

The Python AiBot package also declares version 1, matching the project-wide V1 reference.

# 2. Components

### Arduino firmware

Directory: firmware/SynROV_Firmware/.

Main responsibilities:

motor, servo, ESC, and peripheral I/O;

telemetry and diagnostics;

battery, current, IMU, sonar, and GPS handling;

collision guard and safety routines;

configuration and EEPROM persistence with magic/schema/CRC protection;

robot-specific control for Manipulator, Vehicle, and Drone.

Relevant V1 references:

FIRMWARE_VERSION = 1;

EEPROM_SCHEMA_ID = 1;

additional persistent blocks also use V1 schema identifiers.

Serial0 operates at 115200 baud for communication with the station. Serial1 is reserved for GPS when available.

### Processing

Directory: software/processing/SynROV/.

The Processing application is the main operator station and manages:

serial connection and handshake;

robot selection;

simulation and 3D models;

keyboard, joystick, and Leap Motion input;

maps, environment, and diagnostics;

persistent configuration;

WebSocket server.

V1 files:

data/v1/synrov.json
data/v1/joystick.json
data/v1/manipulator.json
data/v1/vehicle.json
data/v1/drone.json

Missing or invalid files are recreated with defaults. An existing file must declare the correct schema and version: 1.

### Web console

File: software/web/SynROV.html.

The console uses WebSocket, sends V1 JSON commands, and accepts only explicit V1 state packets. Browser preferences use synrov.web.settings.v1 and version: 1.

Default endpoint:

ws://localhost:9000/

### Python AiBot

Directory: software/python/synrov_aibot/.

There are two main execution modes:

Tkinter interface (python -m synrov_aibot);

dedicated runtime (python -m synrov_aibot --modern ...).

The dedicated runtime uses RobotAIRegistry and keeps three independent instances:

ManipulatorAI;

VehicleAI;

DroneAI.

Persistence is stored under:

software/python/synrov_aibot/synrov_multimodal_data/

This directory contains models, datasets, frames, profiles, and learned commands.

# 3. V1 contracts

| Area | Schema / version |
| --- | --- |
| WebSocket command | synrov.control.v1, version: 1 |
| WebSocket state | synrov.state.v1, version: 1 |
| Processing configuration | synrov.config.v1, version: 1 |
| Robot configuration | synrov.robot_config.v1, version: 1 |
| Joystick | synrov.joystick.v1, version: 1 |
| Web configuration | synrov.web.settings.v1, version: 1 |
| AiBot package | version = "1" / __version__ = "1" |
| AiBot dataset | synrov.training.v1 |
| AiBot snapshot | synrov.snapshot.v1 |
| Learned commands | synrov.commands.v1, version: 1 |
| AiBot model | synrov.model.v1, version: 1 |

Processing rejects WebSocket commands that do not include the synrov.control.v1 schema and numeric version 1. The Web console and Python reject state packets without synrov.state.v1 / version: 1.

# 4. WebSocket communication

### Command envelope

Minimal example:

{
  "schema": "synrov.control.v1",
  "version": 1,
  "source": "client",
  "control": {
    "robot": "Vehicle",
    "drive": {
      "throttle": 0.2,
      "steer": 0.0
    }
  }
}

The Python control_envelope() helper forces schema and version to the V1 values, preventing callers from replacing those fields with another version.

### State

Every JSON object sent by Processing through sendWebSocketJson() receives the following metadata when needed:

{
  "schema": "synrov.state.v1",
  "version": 1
}

V1 clients must require both fields.

# 5. Manipulator

### HOME

The Processing reference pose is:

| Joint | HOME |
| --- | --- |
| Base | 180 deg |
| Upper | 150 deg |
| Fore | 70 deg |
| Forearm roll | 90 deg |
| Wrist pitch | 95 deg |
| Wrist rotation | 130 deg |
| Grip | 0 |

The dedicated Python runtime uses the same reference when updating its local state during the HOME action.

### Unified control

{
  "schema": "synrov.control.v1",
  "version": 1,
  "control": {
    "robot": "Manipulator",
    "manipulator": {
      "angles": [180, 150, 70, 90, 95, 130, 0],
      "neutral": [false, false, false, false, false, false, false],
      "duty": [0, 0, 0, 0]
    }
  }
}

neutral, duty, and autoTorque are optional. Processing routes the pose through the same collision-safe path used by local control.

# 6. Vehicle

V1 control:

{
  "schema": "synrov.control.v1",
  "version": 1,
  "control": {
    "robot": "Vehicle",
    "drive": {
      "throttle": 0.0,
      "steer": 0.0,
      "pivot": 0.0,
      "camPan": 0.0,
      "camTilt": 0.0,
      "lights": false,
      "lidarScan": true
    }
  }
}

throttle and steer are clamped to [-1, 1]. pivot is also accepted: when throttle is effectively zero and pivot is non-zero, Processing performs an in-place rotation. Camera, light, and LiDAR settings can be updated in the same block.

# 7. Drone

V1 control:

{
  "schema": "synrov.control.v1",
  "version": 1,
  "control": {
    "robot": "Drone",
    "flight": {
      "throttle": 0.0,
      "yaw": 0.0,
      "pitch": 0.0,
      "roll": 0.0,
      "strafe": 0.0,
      "forward": 0.0,
      "camPan": 0.0,
      "camTilt": 0.0,
      "cameraStreaming": false
    }
  }
}

All six motion axes are clamped to [-1, 1]. takeoff and land are separate V1 JSON actions.

# 8. Actions and system commands

In addition to the control block, the Processing dispatcher recognizes:

client lifecycle (client);

sync;

mode selection;

camera commands;

joystickType;

toggle;

action;

connect.

All messages must arrive inside the V1 envelope required by the dispatcher.

# 9. AiBot execution

Installation:

cd software/python
python -m pip install -r requirements.txt

Tkinter interface:

python -m synrov_aibot

Dedicated runtime:

python -m synrov_aibot --modern --uri ws://127.0.0.1:9000/

Dedicated runtime with a continuous mission loop:

python -m synrov_aibot --modern --uri ws://127.0.0.1:9000/ --loop

The dedicated runtime receives the active robot and language from Processing, synchronizes the matching AI instance, and cancels previous autonomy when the active robot changes.

# 10. AiBot V1 persistence

### Dataset

dataset.py stores schema_version as:

synrov.training.v1

### Snapshot

perception.py identifies snapshots as:

synrov.snapshot.v1

### Learned commands

learned_commands.json uses this envelope:

{
  "schema": "synrov.commands.v1",
  "version": 1,
  "robots": {
    "Manipulator": {},
    "Vehicle": {},
    "Drone": {}
  }
}

Content without this envelope is not loaded as V1 learned-command memory.

### Models

Trained packages include:

{
  "schema": "synrov.model.v1",
  "version": 1
}

The loader rejects packages without this metadata or packages explicitly associated with a different robot.

# 11. Recommended startup

Flash SynROV_Firmware.ino to an Arduino Mega 2560 or compatible board.

Open software/processing/SynROV/SynROV.pde.

Connect the hardware or enable simulation.

Select the correct robot.

Check battery, current, sensors, and telemetry.

Validate HOME and STOP/HOVER before unrestricted motion.

Open software/web/SynROV.html when the Web console is required.

Start AiBot when voice, vision, learning, or mission features are required.

# 12. Software validation

Python:

cd software/python
python -m compileall -q synrov_aibot

Web:

extract the contents of the <script> block from software/web/SynROV.html;

run node --check on the extracted JavaScript.

Firmware and Processing must be compiled with their respective toolchains and required libraries. Hardware bench testing remains mandatory before real operation.

# 13. Bench checklist

correct robot profile selected;

firmware reporting version 1;

WebSocket using V1 schema and version;

Processing configuration files using the correct schema and version: 1;

HOME confirmed;

motor/ESC directions confirmed at low power;

STOP/HOVER validated;

sensors and battery readings consistent;

Manipulator collision guard validated;

camera and auxiliary servos operating within expected ranges;

AiBot connected to the same Processing endpoint.